# Spill Response Checklist

## North Dakota Department of Environmental Quality – Spill Investigation Program

Below is a checklist of the primary spill response and remediation steps. If the answer is “yes” to all questions, the incident may be recommended for closure with the NDDEQ.

☐  Are all personnel safe?

☐  Has the spill been stopped?

☐  Has the spill been contained?

☐  Has the spill been reported?

☐  If the spill impacts or has potential to impact waters of the state, call 1-833-997-7455 as soon as possible, then report within 24 hours through www.spill.nd.gov.

☐  If the spill has not impacted and does not have potential to impact waters of the state, report within 24 hours through www.spill.nd.gov.

☐  Have livestock been fenced out of the site?

☐  Has the spill been fully delineated?

☐  Horizontally?

☐  Vertically?

☐  Were background samples collected?

☐  Are contaminant concentrations known?

☐  Has the spill volume been updated from the original report, if necessary?

☐  Have a site assessment and work plan been submitted and approved?

☐  Has the work plan been completed?

☐  Have confirmation samples been collected and sent to an NDDEQ certified laboratory?

☐  Were the date and time of samples recorded?

☐  Were the locations of all samples recorded and mapped?

☐  Are final sample results within NDDEQ action levels?

☐  Has the site been seeded?

☐  Has erosion been controlled?

☐  Have noxious and invasive weeds been controlled?

☐  Has 75% or greater beneficial vegetation returned?

☐  Have all trash and debris been removed from the site?

☐  Has a final report been submitted to the NDDEQ spills program?